Problem 1

Let f:{1,2,…,1500}×{-1,0,…,2500}→R, f(x,y)=y⋅sin⁡(x-2)2 be the objective function of a maximization problem.

Each phenotype (x,y)∈{1,2,…,1500}×{-1,0,…,2500}corresponds to a genotype represented as a binary string obtained from the base-2 representation of each phenotype component.

- Write a Python function to randomly generate a population pop, of size dim; the fitness of each individual is stored at the end of each chromosomal representation.

- For a given recombination probability pc, write a recombination function using the multi-point crossover operator with 3 crossover points which, based on population pop, produces a new population popc. The resulting population has the same number of individuals (dim) (asexual recombination is also used, and the fitness of each individual is stored at the end of each chromosomal representation).

Solution Hint

For base-2 representation, the standard binary representation of a natural number can be used.

A phenotype (x,y)∈{1,2,…,1500}×{-1,0,…,2500}corresponds to the binary string:

bin(x),bin(y+1)

Where:

- bin(x) has 11 bits because
1024=210<1500<211=2048

- bin(y+1) has 12 bits because
2048=211<2501<212=4096

According to the requirement, a population contains individuals (binary vectors) together with their fitness values (real numbers) – representation using lists (heterogeneous data).

Fitness function

x=base-10 representation of sir[0:11]
y=base-10 representation of sir[11:23]
fitness(sir)=(y-1)⋅sin⁡(x-2)2